Accessible Brain Morphometry from Point-of-Care
Ultra-Low-Field MRI Using Physics-Constrained Deep Learning

What We Built

A deep learning pipeline that predicts normalized whole brain volume (nWBV) — a clinical biomarker for dementia — directly from a 64 mT Hyperfine bedside MRI scanner. These scanners cost roughly 10× less than hospital 3T MRI systems and require no shielded room, making them suitable for low-resource clinics and point-of-care settings.

How It Works

Stage 1: A 3D Vision Transformer (ViT3D, 4.23M parameters) is pre-trained on physics-simulated 64 mT data generated from 156 healthy subjects (IXI dataset). The simulation uses real measured MR physics at 64 mT — tissue-specific T1/T2 relaxation times, Rician noise, and B0 field inhomogeneity — rather than simple Gaussian blur used by prior work. Stage 2: The model is fine-tuned on 375 OASIS-1 subjects with FreeSurfer-derived nWBV labels and Clinical Dementia Rating (CDR) scores. All layers are updated at a low learning rate (5×10⁻⁵).

Results

On the OASIS-1 test set (n=38), the model achieves Pearson r = 0.892 [95% CI: 0.801–0.943] for nWBV prediction — comparable to established cortical morphometry pipelines. The model correctly orders subjects by dementia severity without CDR as a training label: CDR 0.0 → 0.759, CDR 0.5 → 0.745, CDR 1.0 → 0.711 nWBV (−6.3% per CDR step, consistent with literature). On 23 real Hyperfine 64 mT scans (ds006557, Váša 2025), the model shows expected age-related brain atrophy (Spearman ρ = −0.597, p = 0.003). After lightweight domain adaptation using SynthSeg+ pseudo-labels, absolute error on real hardware is MAE = 0.010 nWBV units — within the clinical acceptability threshold of 0.02. Inference takes 130 ms per scan with no FreeSurfer dependency.

Novelty

This is the first work to apply a 3D Vision Transformer for direct nWBV regression from 64 mT MRI, the first to use physics-based 64 mT simulation (not Gaussian blur) for pre-training, and the first to validate on real Hyperfine hardware with FreeSurfer ground truth (via FastSurfer, n=23). Compared to SynthSeg+ (the leading segmentation-based method), our model achieves equivalent clinical utility at 130 ms vs 2–3 minutes, with no external software dependencies.